UHFCA100 Demo

Manual V1.1

1.	iOS Demo

1.1	Install Demo

Follow the link to download, install or go online and search AppStore for Downloads.

2.2	Bluetooth connection

Reader use Bluetooth for iOS device communication, hence the Bluetooth connectivity of iOS device must be enabled beforehand. Turn on UHFCA100, and add a Bluetooth device name as “A100-XXXXXX” on iOS device.

During the initial launch of demo software, if Bluetooth connectivity of Android device has yet being enabled, the app will notify user to turn on the Bluetooth switch.

2.3	Modules of demo App

2.3.1	Main menu

Click the“UHFCA100app” icon on the top left of the screen to view the main menu.

Descriptions of modules are stated below:

| Module |  | Descriptions |
| --- | --- | --- |
|  | Scanner List | Search and manage available scanners |
|  | Scanner Setting | Access and modify current scanner setting |
|  | Tag List Search | Scan tags, obtain tag EPC number, single tag/multiple tags search and wildcard character search. Export tag list to txt file. |
|  | Tag Write | Issue tags and modify tag category and merchandise information. |
|  | Memory Access | Write tag, read tags, lock tags and select tags |

2.3.2	Scanner list

Scanner list is the list of available scanners and the current connection status.

Follow the following steps to connect a scanner:

(1) Click the Bluetooth icon  on the top right of the screen, app will search and list up all the available scanners.

(2) 	Select a scanner from the list, and click the  icon on the right hand side to connect.

（3）Once device is successfully connected, the icon on the right hand side will transform to ,  and notification sound will be emitted by scanner.

Follow the following steps to remove a scanner:

(1) Press and hold the target scanner

2.3.3	Scanner setting

User can access and modify the reader configurations by using this module. The module interface displays the current reader parameters, as shown below:

(1) RFID power, RF output power, adjustable between 3 ~ 30dBm.

(2) Read speed level, slow, normal and fast.

(3) Finding threshold, the search RF threshold, adjustable between 3 ~ 30dBm.

(4) Reduction unit setting, search RF decline magnitude, adjustable between 1 ~ 6.

(5) Scanner volume level, vibration, mute, low, normal and high

(6) Report only new tag, ON represents sound and tag detection report to host will be delivered by reader for new tag only. OFF represents sound and report will be delivered for all tags (including repeat tags).

(7) Continuous read, ON represents enable continuous read, OFF represent single read.

(8) Menu enable, the function of this command is reserved, this command is currently non-effective.

(9) Language, the UI language of scanner, English and Simplified Chinese is supported.

1.3.4	Tag list search

Tag list search is used for tag search application.

There are two types of tag list, “RFID TAG LIST” and “Search Target”.

Add tag to “Search Target” by click a tag from “RFID TAG LIST”.

Click a tag in “Search Target”, user is able to edit wildcard tag value in the popup window. Click “Delete” to remove tag from “Search Target”. Type in wildcard character in the UID input field, then click “Apply” to start wildcard character search.

Sub-menu of the Tag List Search is shown below:

(1) User Search is used for single tag search. User is required to fill in target EPC number in the popup window to setup search condition. Click “Apply” to perform single tag search.

(2) Single Tag/Multi tag is used for multiple tags search. When the amount of tag in “Search Target” is 1, click “Search” to perform single tag search. When the amount of tags in “Search Target” is more than 1, click “Search” to perform multiple tag search.

(3) Click “Normal” to clear list, exit search mode and return to normal inventory.

(4) Click “HEX/ASCII” to convert the display format for tag data.

(5) Click “Save” to export tag list data to TEXT file.

(6) Click the bin  icon to clear tag list and exit search mode.

1.3.5	Tag Write

This function is used for data writing to a specific memory of a RFID tag. A typical application is used for issue new tag.

(1) Setup RF power, variable between 3 ~ 30dBm

(2) Select the data location from the “Bank” drop down menu. “Offset” is the location of the initial data word, and “Length” is the word length to be written. In the field below, type in hexadecimal character string to be written.

(3) If “Auto Increase” is ticked, App will increase data value to be written by 1, after every successful tag-writing.

(4) If tag is classified in certain category or binding with certain merchandise name, the following information will be required. App will group up the tags according to this information.

(5) Click “Write” or the  icon on the top right of the screen to execute write tag operation. Operation result will be displayed in the “Result Value” field.

1.3.6	Tag memory access

Tag memory access includes read, write and lock operations.

1.3.6.1	Tag filter parameters

Tag filter parameters are the filter condition for target tag matching. Reader will only access tags which satisfy the condition.

① Select the data location to be matched from the “MemoryBank” drop down menu. “Offset” is the location of the initial data word, and “Length” is the word length to be matched.

② In the “Data” input field, type in the hexadecimal character string to be matched. The length of this character string must be identical with the value stated in “Length” (1 word = 2 bytes, 1 byte is formed by 2 hexadecimal characters)

③ Click ”Select” to confirm modification and deliver the configuration to reader. Reader response will be displayed in the “Result” field.

NOTE: if tag filter parameters are not setup, reader will access all tags by default, and perform random operation on the first accessed tag.

1.3.6.2 	Read tag memory

This function is used for data reading from a specific memory of a RFID tag.

Select the data location from the “MemoryBank” drop down menu. “Offset” is the location of the initial data word, and “Length” is the word length to be written. Click “Read” to execute read tag operation.

In the case when tags data is successfully read, the “Read” field will display the operation achievement: “UID” is the target tag EPC number, “DATA” is the obtained data of this tag. In the case when operation is failed, the cause of failure will be displayed in the “Result” field.

1.3.6.3 	Write tag memory

This function is used for data writing to a specific memory of a RFID tag.

① Select the data location from the “Memory Bank” drop down menu. “Offset” is the location of the initial data word, and “Length” is the word length to be written.

② 	In the “Data” input field, type in the hexadecimal character string to be written. The length of this character string must be identical with the value stated in “Length” (1 word = 2 bytes, 1 byte is formed by 2 hexadecimal characters)

③ Click “Write” to execute write tag operation. Operation result will be displayed in the “Result” field.

1.3.6.4	Lock RFID tag

This function is used for locking memory area or authority of a RFID tag.

① In the “Lock” field, user can select authority level for one or more data location, including: “Kill PWD”(kill password), “Access PWD”(access password), “ EPC”(EPC memory bank), “ TID” (TID memory bank) “ User” (user memory bank). There are 5 Authority levels, as stated below:

| Authority levels | Description |
| --- | --- |
| ACCESSIBLE | Unprotected accessible |
| ALWAYS_ACCESSIBLE | Permanent accessible |
| SECURED_ACCESSIBLE | Secured accessible |
| ALWAYS_NOT_ACCESSIBLE | Permanent NOT accessible |
| NO_CHANGE | Keep the current configuration |

② Type in tag access code in the “Access PWD” field. The length of password is 8 digits hexadecimal characters.

Note: all-zero access password will cause lock operation failure.

③ Click “Lock” to execute lock tag operation. Operation result will be displayed in the “Result” field.